ZERO-INPUT AND ZERO-STATE RESPONSES

A.  Solving for the Complete Response Using the Zero State and Zero Input Responses

Consider a network with input x(t), impulse response h(t), and response y(t).  The total response y(t) can be divided into two components:  (1) the response with the input applied and no initial conditions in the circuit (the zero-state response) and (2) the response with no input and all initial conditions present (the zero-input response).  Thus we can write y(t) as

y(t)=y0i(t)+y0s(t)                           (1)

The zero-state response is found through convolution, because convolution gives the output of the system with the input applied and all initial conditions set to zero.

y0s=x(t)∗h(t)                                     (2)

If we have no initial conditions, our job is done.  However, if initial conditions exist, there will be a nonzero zero-input response.  What is the form of this response?  Consider the transfer function when the input is zero.  For example, take a series RL circuit.  Consider the transfer function of this circuit given by

H(s)=I(s)V(s)=1L(s+RL)                         (3)

For the zero-input case, I(s) = H(s) V(s) = H(s) * 0.  So the only case where the output current I(s) can be nonzero is for the case where H(s)  infinity.  This will occur for values of s that are the poles of the transfer function.  In the case of this example, we can have a zero-input response (if initial conditions are present) if s = -R/L.  Thus the zero input response contains terms with complex frequencies that are the poles of the transfer function.  In this case,

i0i(t)=Re(Aest)u(t)=Re(Ae−RtL)u(t)=Ae−RtLu(t).       (4)

What will be the zero-state response?  This can be obtained by convolution or by evaluating V(s)H(s).  Assume the input is a unit step function, so v(t) = u(t) and therefore

V(s)=1s.

Then the frequency-domain representation of the output will be

I0s(s)=V(s)H(s)=1Ls(s+RL)=Bs+Cs+RL         (5)

B and C can be extracted from partial-fraction expansion.  Taking the inverse Laplace transform gives

ios(t)=(B+Ce−RtL)u(t).                  (6)

The total response is therefore the sum of the zero-input and zero-state responses:

i(t)=ioi(t)+i0s(t)=Ae−RtLu(t)+Bu(t)+Ce−RtLu(t)                 (7)

At this point, B and C are known, but A is unknown.  The initial condition i(0+) should be used to find A from the entire solution (equation (7)).

B.  The Difference Between Forced and Natural Responses and the Zero-State and Zero-Input Responses

If the circuit differential equation had been solved, the excitation v(t) would have been placed on the right-hand side of the differential equation.  The differential equation for this circuit is

di(t)dt+RLi(t)=u(t)                (8)

The forced response can be seen to be of form

if(t)=Bu(t)                      (9)

where B is an undetermined constant.  The homogeneous differential equation gives the natural response:

in(t)=De−RtLu(t)                    (9)

The total response is given as the sum of the forced and natural responses:

i(t)=if(t)+in(t)=Bu(t)+De−RtLu(t)             (10)

The coefficients B and D are evaluated from the initial current i(0+) at this point.  Compare this with equation (7), the solution obtained from summing the zero-input and zero-state responses.  Note that D = A + C.  It can be seen that the total solutions are identical, but that the zero-input response comes from both the forced and natural responses, and the zero-state response consists of only part of the natural response.

C.  Which Approach is Best?

The forced/natural response approach is best if you are solving a differential equation to obtain a circuit.  However, with frequency domain methods and using convolution, the best approach is to use the zero-state and zero-input responses.  Note, however, that the forced response term will be contributed by the inverse Laplace transform of X(s) in general, while the natural response comes from the terms generated by H(s).