function encodeexample

%This function encodes a quantization level reading from vector bitcode and outputs

%out a new vector of bits (each level gives a four bit representation).

%Written by Charles Baylis, November 22, 2006

bitcode = [15 0 3 9];

nstop = length(bitcode);

for v = 1:nstop %This "for" loop determines the 4-bit binary code for each of the samples.

residue = bitcode(v)              %Residue is what's left over after we determine each bit.

if (residue >= 8)

bitstream(4*(v-1)+1) = 1;

residue = residue - 8

else

bitstream(4*(v-1)+1) = 0;

end

if (residue >= 4)

bitstream(4*(v-1)+2) = 1;

residue = residue - 4

else

bitstream(4*(v-1)+2) = 0;

end

if (residue >= 2)

bitstream(4*(v-1)+3) = 1;

residue = residue - 2

else

bitstream(4*(v-1)+3) = 0;

end

if (residue >=1)

bitstream(4*(v-1)+4) = 1;

residue = residue -1

else

bitstream(4*(v-1)+4) = 0;

end

end

bitcounter = 1:4*nstop;

bitstream

Program Output:

>> encodeexample

residue =

15

residue =

7

residue =

3

residue =

1

residue =

0

residue =

0

residue =

3

residue =

1

residue =

0

residue =

9

residue =

1

residue =

0

bitstream =

Columns 1 through 12

1     1     1     1     0     0     0     0     0     0     1     1

Columns 13 through 16

1     0     0     1