function quantizeexample

%This function quantizes a vector's values.

%Quantize with L = 16 (4 bits)

myvector = [4.3 2.5 -1.6]

nstop= length(myvector)

mp = 5;                    %Maximum value

L = 16;                    %Number of Levels

deltav = 2*mp/L;           %Coverage of each quantization region

v=1;                       %Loop counter

while (v <= nstop)         %While we haven't reached the end of the vector

upperbound=mp           %Initial upper limit of bin

lowerbound=mp-deltav    %Initial lower limit of bin

i = L-1

while (myvector(v)<lowerbound)  %This loop keeps going until the correct region is found.

upperbound = lowerbound

lowerbound = lowerbound-deltav

i = i-1 %Decrement the quantization level indicator

end

quant(v) = (upperbound+lowerbound)/2;   %The quantized value is the center of the region in which the sample appears.

bitcode(v) = i      %This gives the level of the region (a number 0 to 15)

v = v+1             %Repeat the comparison at the next element in the vector.

end

myvector =

4.3000    2.5000   -1.6000

nstop =

3

upperbound =

5

lowerbound =

4.3750

i =

15

upperbound =

4.3750

lowerbound =

3.7500

i =

14

bitcode =

14

v =

2

The program keeps going…

Final Result:

bitcode =

14    12     5